Using harmonic balance to solve input impedance of HSMS-2822 diode, which uses the series SPICE model at 2 GHz using 5 tone.

Zin= Vin, 1Iin,1

Series SPICE model: vint-vj(t)Rs= Isevj(t)a*VT-1+Cjvj(t)dvj(t)dt

iin(tn)= Isevj(tn)a*VT-1+Cjvj(tn)dvj(tn)dt

You could form the Jacobian now, but we haven’t computed the values yet for vj, vj’, Cj, and Cj’:

∂iin(tn)∂Vm=ejmω0tnISevj/(aVT)aVT+  Cj'(vj) vj  +  jmω0Cj(vj)

DFT :     ∂Iin, k∂Vm= 1Mn=0M-1e-jkω0tn∂iin(tn)∂Vm

∂Iin,k∂Vm= 1Mn=0M-1e-jkω0tnejmω0tnISevj/(aVT)aVT+  Cj'(vj) vj  +  jmω0Cj(vj)

Is=3×10-8

Rs=25 Ω

VT=25 mV

a=1.06

Cj0=0.18 pF=0.18*10-12 F

Vj=0.7 V

m =0.5

FC=0.5

Using Nyquist sampling with exact precision: M = 2(2N+1)

M = 22

Time samples:

tn=n*∆t

∆t=2πω0M

∆t=2π2*π*2*109*22=

| n | tn(seconds) |
| --- | --- |
| 0 | 0 |
| 1 | 2.27272727×10-11 |
| 2 | 4.54545455×10-11 |
| 3 | 6.81818182×10-11 |
| 4 | 9.09090909×10-11 |
| 5 | 1.13636364×10-10 |
| 6 | 1.36363636×10-10 |
| 7 | 1.59090909×10-10 |
| 8 | 1.81818182×10-10 |
| 9 | 2.04545455×10-10 |
| 10 | 2.27272727×10-10 |
| 11 | 2.50000000×10-10 |
| 12 | 2.72727273×10-10 |
| 13 | 2.95454545×10-10 |
| 14 | 3.18181818×10-10 |
| 15 | 3.40909091×10-10 |
| 16 | 3.63636364×10-10 |
| 17 | 3.86363636×10-10 |
| 18 | 4.09090909×10-10 |
| 19 | 4.31818182×10-10 |
| 20 | 4.54545455×10-10 |
| 21 | 4.77272727×10-10 |

Voltage Source

v₀ = v_s(t₀) = 0.1 sin(2·π·0 / 22) = 0.000000000
v₁ = v_in(t₁) = 0.1 sin(2·π·1 / 22) = 0.028173256
v₂ = v_in(t₂) = 0.1 sin(2·π·2 / 22) = 0.054064082
v₃ = v_in(t₃) = 0.1 sin(2·π·3 / 22) = 0.075574957
v₄ = v_in(t₄) = 0.1 sin(2·π·4 / 22) = 0.090963200
v₅ = v_in(t₅) = 0.1 sin(2·π·5 / 22) = 0.098982144
v₆ = v_in(t₆) = 0.1 sin(2·π·6 / 22) = 0.098982144
v₇ = v_in(t₇) = 0.1 sin(2·π·7 / 22) = 0.090963200
v₈ = v_in(t₈) = 0.1 sin(2·π·8 / 22) = 0.075574957
v₉ = v_in(t₉) = 0.1 sin(2·π·9 / 22) = 0.054064082
v₁₀ = v_in(t₁₀) = 0.1 sin(2·π·10 / 22) = 0.028173256
v₁₁ = v_in(t₁₁) = 0.1 sin(2·π·11 / 22) = 0.000000000
v₁₂ = v_in(t₁₂) = 0.1 sin(2·π·12 / 22) = -0.028173256
v₁₃ = v_in(t₁₃) = 0.1 sin(2·π·13 / 22) = -0.054064082
v₁₄ = v_in(t₁₄) = 0.1 sin(2·π·14 / 22) = -0.075574957
v₁₅ = v_in(t₁₅) = 0.1 sin(2·π·15 / 22) = -0.090963200
v₁₆ = v_in(t₁₆) = 0.1 sin(2·π·16 / 22) = -0.098982144
v₁₇ = v_in(t₁₇) = 0.1 sin(2·π·17 / 22) = -0.098982144
v₁₈ = v_in(t₁₈) = 0.1 sin(2·π·18 / 22) = -0.090963200
v₁₉ = v_in(t₁₉) = 0.1 sin(2·π·19 / 22) = -0.075574957
v₂₀ = v_in(t₂₀) = 0.1 sin(2·π·20 / 22) = -0.054064082
v₂₁ = v_in(t₂₁) = 0.1 sin(2·π·21 / 22) = -0.028173256
v₂₂ = v_in(t₂₂) = 0.1 sin(2·π·22 / 22) = 0.000000000

For initial newton iteration, we find fundamental components

Vs(t) = 0.1sin(wt) V  where f0 = 2 GHz, taking the fourier transform of Vs(t) (with 5 harmonics) we get V_in,k values calculated as shown below:

ω0​tn= 2πnM

vintn= k = -NNVin,  k​ejkω0​tn​

n = 1

k=-5→ Vin,-5ej(-5)⋅2π/22=0* e-j10π/22=0

k=-4→ 0

k=-3→ 0

k=-2→ 0

k=-1→ j0.05e-jπ/11

k=0→ 0

k=1→ -j0.05ejπ/11

k=2→ 0

k=3→ 0

k=4→ 0

k=5→ 0

Using Eulers formula to get complex numbers:

ejθ=cos(θ)+jsin(θ)

sum all: Vin,kejkω0​tn=Vin,k[coskω0​tn+jsinkω0​tn]

j0.05e-jπ/11+j0.05ejπ/11

=j0.05*cos-π11+jsin-π11- j0.05*[cosπ11+jsinπ11]

≈0.02817

If  you add all of the expressions with its corresponding k value, you get (no need imaginary components, they canceled when evaluated):

vin(t0)≈0.000000000 V

vin(t1)≈0.028173256 V

vin(t2)≈0.054064082 V

vin(t3)≈0.075574957 V

vin(t4)≈0.090963200 V

vin(t5)≈0.098982144 V

vin(t6)≈0.098982144 V

vin(t7)≈0.090963200 V

vin(t8)≈0.075574957 V

vin(t9)≈0.054064082 V

vin(t10)≈0.028173256 V

vin(t11)≈0.000000000 V

vin(t12)≈-0.028173256 V

vin(t13)≈-0.054064082 V

vin(t14)≈-0.075574957 V

vin(t15)≈-0.090963200 V

vin(t16)≈-0.098982144 V

vin(t17)≈-0.098982144 V

vin(t18)≈-0.090963200 V

vin(t19)≈-0.075574957 V

vin(t20)≈-0.054064082 V

vin(t21)≈-0.028173256 V

vin't= k = -NNjkω0Vk​ejkω0t​

vin'tn= k= -NNjkw0Vkej*k*2*π*nM

vin't0= k= -55jk2*π*2*109Vkej*k*2*π*022

vin't1= k= -55jk2*π*2*109Vkej*k*2*π*122

…

vin't21= k= -55jk2*π*2*109Vkej*k*2*π*2122

At n = 0:

k=-5→j*-5*2π*2*109*0*ej*-5*π*022

k=-4→j*-4*2π*2*109*0*ej*-4*π*022

k=-3→j*-3*2π*2*109*0*ej*-3*π*022

k=-2→j*-2*2π*2*109*0*ej*-2*π*022

k=-1→j*-1*2π*2*109*j0.05*ej*-1*π*022

k=0→j*0*2π*2*109*0*ej*0*π*022

k=1→j*1*2π*2*109*-j0.05*ej*1*π*022

k=2→j*2*2π*2*109*0*ej*2*π*022

k=3→j*3*2π*2*109*0*ej*3*π*022

k=4→j*4*2π*2*109*0*ej*4*π*022

k=5→j*5*2π*2*109*0*ej*5*π*022

If  you add all of the expressions at each time sample with its corresponding k value, you get:

Example:

vin't0≈R{1.25663706×109+j0} ≈1.25663706×109 V

Values for each V'd(tn) , n = 0 to 21

n=0: vin'(t0)≈1.25663706×109

n=1: vin'(t1)≈1.20573443×109

n=2: vin'(t2)≈1.05715037×109

n=3: vin'(t3)≈8.22922268×108

n=4: vin'(t4)≈5.22025901×108

n=5: vin'(t5)≈1.78838100×108

n=6: vin'(t6)≈-1.78838100×108

n=7: vin'(t7)≈-5.22025901×108

n=8: vin'(t8)≈-8.22922268×108

n=9: vin'(t9)≈-1.05715037×109

n=10: vin'(t10)≈-1.20573443×109

n=11: vin'(t11)≈-1.25663706×109

n=12: vin'(t12)≈-1.20573443×109

n=13: vin'(t13)≈-1.05715037×109

n=14: vin'(t14)≈-8.22922268×108

n=15: vin'(t15)≈-5.22025901×108

n=16: vin'(t16)≈-1.78838100×108

n=17: vin'(t17)≈1.78838100×108

n=18: vin'(t18)≈5.22025901×108

n=19: vin'(t19)≈8.22922268×108

n=20: vin'(t20)≈1.05715037×109

n=21: vin'(t21)≈1.20573443×109

First find initial conditions for newtons iteration by finding fundamental components of Vj,k where k = -1 and 1 and 0 for all other k.

Vj,-1=Vin,-1ZCZC+ Rs

Vj,1=Vin,1ZCZC+ Rs

ZC=1j*ω0*Cj

ZC=1j*2*π*2*109*0.18*10-12= -j442.1

Vj,-1=j0.05*-j442.1-j442.1+ 25=0.002818+j0.04987

Vj,1=-j0.05*-j442.1-j442.1+ 25= -0.002818-j0.04987

ω0​tn= 2πnM

vjtn= k = -NNVj,  k​ejkω0​tn​

k=-5→ V-5ej-5⋅2π*(1)/22=0* e-j10π/22=0

k=-4→ 0

k=-3→ 0

k=-2→ 0

k=-1→ 0.002818+j0.04987ej(-1)⋅π/11

k=0→ 0

k=1→ - 0.002818-j0.04987ej(1)⋅π/11

k=2→ 0

k=3→ 0

k=4→ 0

k=5→ 0

Using Eulers formula to get complex numbers:

ejθ=cos(θ)+jsin(θ)

sum all: Vj,kejkω0​tn=Vj,k[coskω0​tn+jsinkω0​tn]

At n = 1:

(0.002818+j0.04987)ej(-1)⋅π/11 (- 0.002818-j0.04987)ej(1)⋅π/11

=0.002818+j0.04987*cos-π11+jsin-π11+ (- 0.002818-j0.04987)*[cosπ11+jsinπ11]

≈0.028100-j0.001588

If  you add all of the expressions with its corresponding k value, you get: Vj(tn) values:

n=0:  0.000000  +  j0.000000

n=1:  0.028100  -  j0.001588

n=2:  0.053924  -  j0.003047

n=3:  0.075378  -  j0.004259

n=4:  0.090727  -  j0.005127

n=5:  0.098725  -  j0.005579

n=6:  0.098725  -  j0.005579

n=7:  0.090727  -  j0.005127

n=8:  0.075378  -  j0.004259

n=9:  0.053924  -  j0.003047

n=10: 0.028100  -  j0.001588

n=11: ≈0.000000  +  j0.000000 (numerically 5.64×10-17)

n=12: -0.028100  +  j0.001588

n=13: -0.053924  +  j0.003047

n=14: -0.075378  +  j0.004259

n=15: -0.090727  +  j0.005127

n=16: -0.098725  +  j0.005579

n=17: -0.098725  +  j0.005579

n=18: -0.090727  +  j0.005127

n=19: -0.075378  +  j0.004259

n=20: -0.053924  +  j0.003047

n=21: -0.028100  +  j0.001588

vj'tn= k = -NNjkω0Vj,k​ejkω0t​

vj'tn= k= -NNjkw0Vj,kej*k*2*π*nM

vj't0= k= -55jk2*π*2*109Vj,kej*k*2*π*022

vj't1= k= -55jk2*π*2*109Vj,kej*k*2*π*122

…

vj't21= k= -55jk2*π*2*109Vj,kej*k*2*π*2122

At n = 0:

k=-5→j*-5*2π*2*109*0*ej*-5*π*022

k=-4→j*-4*2π*2*109*0*ej*-4*π*022

k=-3→j*-3*2π*2*109*0*ej*-3*π*022

k=-2→j*-2*2π*2*109*0*ej*-2*π*022

k=-1→j*-1*2π*2*109*j0.05*ej*-1*π*022

k=0→j*0*2π*2*109*0*ej*0*π*022

k=1→j*1*2π*2*109*-j0.05*ej*1*π*022

k=2→j*2*2π*2*109*0*ej*2*π*022

k=3→j*3*2π*2*109*0*ej*3*π*022

k=4→j*4*2π*2*109*0*ej*4*π*022

k=5→j*5*2π*2*109*0*ej*5*π*022

If  you add all of the expressions at each time sample with its corresponding k value, you get:

Example:

vj't0≈R{1.25663706×109+j0} ≈1.2547×109 V

Values for each Vj'(tn) , n = 0 to 21

n=0,         1.2547e9-j7.082e7

n=1,  1.2048e9-j6.797e7

n=2,  1.0551e9-j5.953e7

n= 3,  8.213e8-j4.638e7

n=4,  5.214e8-j2.941e7

n=5,  1.785e8-j1.009e7

n=6,  -1.785e8+j1.009e7

n=7,  -5.214e8+j2.941e7

n=8,  -8.213e8+j4.638e7

n=9,  -1.0551e9+j5.953e7

n=10,  -1.2048e9+j6.797e7

n=11       -1.2547e9+j7.082e7

n=12,  -1.2048e9+j6.797e7

n=13,  -1.0551e9+j5.953e7

n=14,  -8.213e8+j4.638e7

n=15,  -5.214e8+j2.941e7

n=16,  -1.785e8+j1.009e7

n=17,  1.785e8-j1.009e7

n=18,  5.214e8-j2.941e7

n=19,  8.213e8-j4.638e7

n=20,  1.0551e9-j5.953e7

n=21,  1.2048e9-j6.797e7

Capacitor

Computing Cj

Cj(vj,n) =

{ Cj0 * (1 - vnVj)-m,                                   for vj,n ≤ Fc * Vj

{ Cj0 * (1 - Fc)(-m-1) * 1 - Fc*(1 + m) + m*vnVj,     for vj,n > Fc * Vj

Cjvj,0=

{ 0.18*10-12* 1 - 00.7-0.5,                                   for 0 ≤ 0.5 * 0.7

{ 0.18*10-12 * 1 - 0.5-0.5-1 * 1 - 0.5*1 + 0.5+ 0.5*00.7,     for  0> 0.5 * 0.7

Cjvj,1=

{ 0.18*10-12* 1 - 0.028100 0.7-0.5,                                   for 0.028100 ≤ 0.5 * 0.7

{ 0.18*10-12 * 1 - 0.5-0.5-1 * 1 - 0.5*1 + 0.5+ 0.5*0.028100  0.7,    0.028100  -  j0.001588> 0.5 * 0.7

Cj(vj,0)≈1.80×10-13

Cj(vj,1)≈1.8373×10-13

Cj(vj,2)≈1.8736×10-13

Cj(vj,3)≈1.9055×10-13

Cj(vj,4)≈1.9294×10-13

Cj(vj,5)≈1.9422×10-13

Cj(vj,6)≈1.9422×10-13

Cj(vj,7)≈1.9294×10-13

Cj(vj,8)≈1.9055×10-13

Cj(vj,9)≈1.8736×10-13

Cj(vj,10)≈1.8373×10-13

Cj(vj,11)≈1.8000×10-13

Cj(vj,12)≈1.7649×10-13

Cj(vj,13)≈1.7344×10-13

Cj(vj,14)≈1.7103×10-13

Cj(vj,15)≈1.6936×10-13

Cj(vj,16)≈1.6851×10-13

Cj(vj,17)≈1.6851×10-13

Cj(vj,18)≈1.6936×10-13

Cj(vj,19)≈1.7103×10-13

Cj(vj,20)≈1.7344×10-13

Cj(vj,21)≈1.7649×10-13

Derivative of Cj : Cj’ values

C'jvn=dCjdV

{ Cj0*m*1Vj*(1 - vnVj)(-m-1),                                   for vn ≤ Fc * Vj

{ Cj0 * 1 - Fc-m-1 * mVj,     for vn > Fc * Vj

C'jv0=dCjdV

{ 0.18*10-12*0.5*10.7*1 - 00.7-0.5-1,                                   for 0 ≤ 0.5* 0.7

{ 0.18*10-12 * 1 - 0.5-0.5-1 * 0.50.7,     for 0 > 0.5* 0.7

C'jv1=dCjdV

{ 0.18*10-12*0.5*10.7*1 - 0.028000.7-0.5-1,    for 0.028 ≤ 0.5* 0.7

{ 0.18*10-12 * 1 - 0.5-0.5-1 * 0.50.7              ,     for 0.028 > 0.5* 0.7

n = 0, 1.2718e-13 + j0

n = 1, 1.2983e-13 + j0

n = 2, 1.3483e-13 + j0

n = 3, 1.4217e-13 + j0

n = 4, 1.5170e-13 + j0

n = 5, 1.6308e-13 + j0

n = 6, 1.7587e-13 + j0

n = 7, 1.8956e-13 + j0

n = 8, 2.0354e-13 + j0

n = 9, 2.1711e-13 + j0

n = 10, 2.2954e-13 + j0

n = 11, 2.4014e-13 + j0

n = 12, 2.4828e-13 + j0

n = 13, 2.5345e-13 + j0

n = 14, 2.5532e-13 + j0

n = 15, 2.5379e-13 + j0

n = 16, 2.4893e-13 + j0

n = 17, 2.4103e-13 + j0

n = 18, 2.3054e-13 + j0

n = 19, 2.1795e-13 + j0

n = 20, 2.0375e-13 + j0

n = 21, 1.8840e-13 + j0

I use the guessed Vn values and plug them into the equation then subtract my linear Iin,k values are shown in yellow highlight with the non-linear Iin,k values from the equation highlighted in blue to get the residuals as shown below shown in red highlight.

Rk=  Vin,k-Vj,kRs -Iin(Vj,k)

My first try. I did Iin,k values in yellow minus Iin,k values in yellow at first, but didn’t do the actual Iin,k values.

Iin,n= Vin,n-Vj,nRs

Iin,1= Vin,1-Vj,1Rs=-j0.05-( - 0.002818-j0.04987)25=

Iin,1=1.1272 ×10-4-j5.2×10-6

Iin,-1= Vin,-1-Vj,-1Rs=j0.05-( - 0.002818-j0.04987)25

Iin,-1=1.1272 ×10-4+j5.2×10-6

iintn= k = -55Iin,  k​ejkω0​tn​

k=-5→ Iin,-5ej(-5)⋅2π/22=0* e-j10π/22=0

k=-4→ 0

k=-3→ 0

k=-2→ 0

k=-1→ 1.1272 ×10-4-j5.2×10-6ej(-1)⋅π/11

k=0→ 0

k=1→ 1.1272 ×10-4+j5.2×10-6ej(1)⋅π/11

k=2→ 0

k=3→ 0

k=4→ 0

k=5→ 0

iin(t0),         0.00022544+j0

iin(t1),  0.00021924+j0

iin(t2),  0.00019527+j0

iin(t3),  0.00015549+j0

iin(t4),  0.00010311+j0

iin(t5),  0.00004238+j0

iin(t6),  -0.00002179+j0

iin(t7),  -0.00008419+j0

iin(t8),  -0.00013977+j0

iin(t9),  -0.00018403+j0

iin(t10),  -0.00021338+j0

iin(t11),  -0.00022544+j0

iin(t12),  -0.00021924+j0

iin(t13),  -0.00019527+j0

iin(t14),  -0.00015549+j0

iin(t15),  -0.00010311+j0

iint16,  -0.00004238+j0

iint17,  0.00002179+j0

iint18,  0.00008419+j0

iint19,  0.00013977+j0

iint20,  0.00018403+j0

iint21,  0.00021338+j0

iin(tn)= Isevj(tn)a*VT-1+Cjvj(tn)dvj(tn)dt

n = 0, 2.25846e-04 + j0

n = 1, 2.21415e-04 + j0

n = 2, 1.97883e-04 + j0

n = 3, 1.56984e-04 + j0

n = 4, 1.01489e-04 + j0

n = 5, 3.58830e-05 + j0

n = 6, -3.34535e-05 + j0

n = 7, -9.97085e-05 + j0

n = 8, -1.56013e-04 + j0

n = 9, -1.97484e-04 + j0

n = 10, -2.21301e-04 + j0

n = 11, -2.25846e-04 + j0

n = 12, -2.12655e-04 + j0

n = 13, -1.83023e-04 + j0

n = 14, -1.40495e-04 + j0

n = 15, -8.83333e-05 + j0

n = 16, -3.01083e-05 + j0

n = 17, 3.00498e-05 + j0

n = 18, 8.82753e-05 + j0

n = 19, 1.40439e-04 + j0

n = 20, 1.82970e-04 + j0

n = 21, 2.12616e-04 + j0

Take the DTF of my i_in(tn) equation to get I_in,k values
Iin, k=1Mn=0M-1iin(tn)*e-j2πkn/M

Iin, k=1Mn=0M-1 Isevj(tn)a*VT-1+Cjvj(tn)dvj(tn)dt *e-j2πkn/M

k = -5, 2.74333868e-08 + j1.04095281e-08

k = -4, 3.06797224e-08 + j-1.20990069e-08

k = -3, -1.90648453e-07 + j-7.56176288e-08

k = -2, -1.51223218e-07 + j4.04807398e-06

k = -1, 1.13151512e-04 + j2.36330930e-07

k =  0, 2.46804650e-07 + j0

k =  1, 1.13151512e-04 + j-2.36330930e-07

k =  2, -1.51223218e-07 + j-4.04807398e-06

k =  3, -1.90648453e-07 + j7.56176288e-08

k =  4, 3.06797224e-08 + j1.20990069e-08

k =  5, 2.74333868e-08 + j-1.04095281e-08

Rn= vin,n-vj,nRs-Iinvj,n

Rk= Vin,k-Vj,kRs-Iin(Vj,k)

R-5= Vin,-5-Vj,-5Rs-Iin(Vj,-5)

R-5= Vin,-5-Vj,-5Rs-Iin(Vj,-5)

Example when k = -5:

R-5=  Iin, guessed(Vj,-5) -  Iin(Vj,-5)

R-5= 0-(2.74333868e-08 + j1.04095281e-08)

R-5=-2.74333868e-08- j1.04095281e-08

k = -5, -2.74333868e-08 + j-1.04095281e-08

k = -4, -3.06797224e-08 + j1.20990069e-08

k = -3,  1.90648453e-07 + j7.56176288e-08

k = -2,  1.51223218e-07 + j-4.04807398e-06

k = -1, -2.25871512e-04 + j4.96366907e-06

k =  0, -2.46804650e-07 + j0

k =  1, -4.31512000e-07 + j-4.96366907e-06

k =  2,  1.51223218e-07 + j4.04807398e-06

k =  3,  1.90648453e-07 + j-7.56176288e-08

k =  4, -3.06797224e-08 + j-1.20990069e-08

k =  5, -2.74333868e-08 + j1.04095281e-08

Now do one newton iteration to get smaller residuals (you don’t have to do the newton iteration if your residual is very small, meaning good approximation, and you don’t have to form the Jacobian):

calculate time domain Jacobian first (11x22 matrice), that way you can easily calculate the frequency domain Jacobian:

∂iin(tn)∂Vm=ejmω0tnISevj(tn)/(aVT)aVT  +  Cj'(vj(tn)) vj(tn) +  jmω0 (Cj(Vj(tn))

∂iin(tn)∂Vm=ejm2πnMISevj(tn)/(aVT)aVT  +  Cj'(vj(tn)) vj(tn) +  jmω0 (Cj(Vj(tn))

∂iin(t0)∂V-5=ej(-5)2π*022ISevj(t0)/(aVT)aVT  +  Cj'(vj(t0)) vj(t0) +  j(-5)(2π*2*109) (Cj(Vj(t0))

∂iin(t0)∂V-5=ej(-5)2π*0223×10-8e0/(1.06*25*10-3)1.06*25*10-3  + (1.2718*10-13) (1.2547e9-j7.082e7 )+  j(-5)(2π*2*109) (1.80×10-13)= 1.60704821e-04 + j-1.13187404e-02

n = 0, 1.60704821e-04 + j-1.13187404e-02, 1.60704821e-04 + j-9.05679373e-03, 1.60704821e-04 + j-6.79484702e-03, 1.60704821e-04 + j-4.53290031e-03, 1.60704821e-04 + j-2.27095360e-03, 1.60704821e-04 + j-9.00688760e-06, 1.60704821e-04 + j2.25293982e-03, 1.60704821e-04 + j4.51488653e-03, 1.60704821e-04 + j6.77683324e-03, 1.60704821e-04 + j9.03877995e-03, 1.60704821e-04 + j1.13007267e-02

n = 1, -1.14126029e-02 + j-1.80221468e-03, -8.34239382e-03 + j-2.32767214e-03, -5.32839195e-03 + j-2.46153474e-03, -2.44464122e-03 + j-2.19388420e-03, 2.21729330e-05 + j-1.55991789e-03, 2.18336252e-03 + j-6.36565801e-04, 3.63343692e-03 + j4.60762280e-04, 4.30988027e-03 + j1.56622925e-03, 4.17602438e-03 + j2.60538515e-03, 3.23255004e-03 + j3.50427098e-03, 1.51628259e-03 + j4.19627159e-03

n = 2, -1.15604798e-02 + j2.14466984e-03, -9.56176565e-03 + j-2.21342946e-04, -7.31815483e-03 + j-2.16923005e-03, -4.84175174e-03 + j-3.97190136e-03, -2.23872122e-03 + j-5.34778732e-03, 1.68904523e-05 + j-6.17775950e-03, 2.12519299e-03 + j-6.41757342e-03, 4.08503982e-03 + j-6.09793258e-03, 5.81406054e-03 + j-5.31688673e-03, 7.24885964e-03 + j-4.22017545e-03, 8.34921023e-03 + j-2.98682419e-03

n = 3, -9.62217511e-03 + j4.87254000e-03, -8.95492970e-03 + j3.16614869e-03, -7.95620903e-03 + j9.91089548e-04, -6.68913296e-03 + j-1.27248863e-03, -5.22925346e-03 + j-3.45202062e-03, -3.65935032e-03 + j-5.39177931e-03, -2.06326552e-03 + j-6.96279377e-03, -5.20733992e-04 + j-8.07357158e-03, 8.64480585e-04 + j-8.67257958e-03, 2.02618318e-03 + j-8.75191802e-03, 2.92297213e-03 + j-8.34436655e-03

n = 4, -6.08211873e-03 + j6.14106719e-03, -6.61590667e-03 + j5.30169096e-03, -7.09711655e-03 + j4.07004474e-03, -7.45522295e-03 + j2.54967256e-03, -7.63824824e-03 + j8.85152552e-04, -7.61428962e-03 + j-7.69004672e-04, -7.37312957e-03 + j-2.26291314e-03, -6.92666993e-03 + j-3.46603104e-03, -6.30704396e-03 + j-4.27629063e-03, -5.56135064e-03 + j-4.62784682e-03, -4.74576241e-03 + j-4.49692718e-03

n = 5, -2.33178243e-03 + j6.47067949e-03, -3.51271006e-03 + j6.18920599e-03, -4.62697386e-03 + j5.51358821e-03, -5.60125503e-03 + j4.50117914e-03, -6.36816454e-03 + j3.23716746e-03, -6.87164917e-03 + j1.82603766e-03, -7.07115435e-03 + j3.83450218e-04, -6.94309009e-03 + j-1.00979564e-03, -6.48399744e-03 + j-2.27392847e-03, -5.71130973e-03 + j-3.34066786e-03, -4.66253249e-03 + j-4.15601522e-03

n = 6, 2.11830448e-03 + j5.93561170e-03, 1.02258539e-03 + j6.23581414e-03, -2.42696925e-04 + j6.13775326e-03, -1.48179384e-03 + j5.64995999e-03, -2.63166195e-03 + j4.81468380e-03, -3.63537009e-03 + j3.70282298e-03, -4.44636293e-03 + j2.40602480e-03, -5.02918214e-03 + j1.02924723e-03, -5.36015412e-03 + j-3.21128397e-04, -5.42898460e-03 + j-1.55456075e-03, -5.23838124e-03 + j-2.59320748e-03

n = 7, 5.92469369e-03 + j4.63847103e-03, 5.07957566e-03 + j5.31457168e-03, 4.05992171e-03 + j5.63971464e-03, 2.92371403e-03 + j5.60022960e-03, 1.73734321e-03 + j5.22589071e-03, 5.70618496e-04 + j4.58648188e-03, -5.13205305e-04 + j3.78295716e-03, -1.45126731e-03 + j2.93650629e-03, -2.19355915e-03 + j2.17285059e-03, -2.71003394e-03 + j1.60607209e-03, -2.99218860e-03 + j1.32301939e-03

n = 8, 8.30449922e-03 + j2.83754680e-03, 7.38484536e-03 + j3.80065434e-03, 6.31960720e-03 + j4.43443216e-03, 5.18231791e-03 + j4.71666510e-03, 4.05313734e-03 + j4.67802330e-03, 3.00966571e-03 + j4.39438036e-03, 2.12156584e-03 + j3.97962831e-03, 1.44392247e-03 + j3.56869299e-03, 1.01101859e-03 + j3.29772840e-03, 8.34675678e-04 + j3.27925101e-03, 9.00316677e-04 + j3.57518955e-03

n = 9, 8.82860269e-03 + j9.95296253e-04, 8.00681052e-03 + j2.12133178e-03, 7.01643073e-03 + j3.00922620e-03, 5.93177045e-03 + j3.62078593e-03, 4.84266310e-03 + j3.95835907e-03, 3.84378443e-03 + j4.05987180e-03, 3.02175582e-03 + j3.98981975e-03, 2.44426424e-03 + j3.82816169e-03, 2.15023446e-03 + j3.66177226e-03, 2.14152785e-03 + j3.57455574e-03, 2.37823742e-03 + j3.63971739e-03

n = 10, 7.47313124e-03 + j-5.98954104e-04, 6.86596250e-03 + j6.22964107e-04, 6.13134099e-03 + j1.69157484e-03, 5.31449713e-03 + j2.56176361e-03, 4.46743367e-03 + j3.22149694e-03, 3.64590287e-03 + j3.68736242e-03, 2.90598220e-03 + j3.99962455e-03, 2.30070667e-03 + j4.21458507e-03, 1.87692447e-03 + j4.39513079e-03, 1.67303265e-03 + j4.60397183e-03, 1.71718957e-03 + j4.89575410e-03

n = 11, 4.32910795e-03 + j-1.67782976e-03, 4.01279393e-03 + j-8.07187544e-04, 3.61488057e-03 + j1.58590421e-05, 3.17041441e-03 + j7.72177086e-04, 2.71025155e-03 + j1.43635256e-03, 2.25948185e-03 + j1.98994024e-03, 1.83682763e-03 + j2.42568471e-03, 1.45427896e-03 + j2.74254596e-03, 1.11838963e-03 + j2.94310608e-03, 8.31195324e-04 + j3.03138848e-03, 5.91498637e-04 + j3.01183990e-03

n = 12, 4.32910795e-03 + j1.67782976e-03, 4.01279393e-03 + j8.07187544e-04, 3.61488057e-03 + j-1.58590421e-05, 3.17041441e-03 + j-7.72177086e-04, 2.71025155e-03 + j-1.43635256e-03, 2.25948185e-03 + j-1.98994024e-03, 1.83682763e-03 + j-2.42568471e-03, 1.45427896e-03 + j-2.74254596e-03, 1.11838963e-03 + j-2.94310608e-03, 8.31195324e-04 + j-3.03138848e-03, 5.91498637e-04 + j-3.01183990e-03

n = 13, 7.47313124e-03 + j5.98954104e-04, 6.86596250e-03 + j-6.22964107e-04, 6.13134099e-03 + j-1.69157484e-03, 5.31449713e-03 + j-2.56176361e-03, 4.46743367e-03 + j-3.22149694e-03, 3.64590287e-03 + j-3.68736242e-03, 2.90598220e-03 + j-3.99962455e-03, 2.30070667e-03 + j-4.21458507e-03, 1.87692447e-03 + j-4.39513079e-03, 1.67303265e-03 + j-4.60397183e-03, 1.71718957e-03 + j-4.89575410e-03

n = 14, 8.82860269e-03 + j-9.95296253e-04, 8.00681052e-03 + j-2.12133178e-03, 7.01643073e-03 + j-3.00922620e-03, 5.93177045e-03 + j-3.62078593e-03, 4.84266310e-03 + j-3.95835907e-03, 3.84378443e-03 + j-4.05987180e-03, 3.02175582e-03 + j-3.98981975e-03, 2.44426424e-03 + j-3.82816169e-03, 2.15023446e-03 + j-3.66177226e-03, 2.14152785e-03 + j-3.57455574e-03, 2.37823742e-03 + j-3.63971739e-03

n = 15, 8.30449922e-03 + j-2.83754680e-03, 7.38484536e-03 + j-3.80065434e-03, 6.31960720e-03 + j-4.43443216e-03, 5.18231791e-03 + j-4.71666510e-03, 4.05313734e-03 + j-4.67802330e-03, 3.00966571e-03 + j-4.39438036e-03, 2.12156584e-03 + j-3.97962831e-03, 1.44392247e-03 + j-3.56869299e-03, 1.01101859e-03 + j-3.29772840e-03, 8.34675678e-04 + j-3.27925101e-03, 9.00316677e-04 + j-3.57518955e-03

n = 16, 5.92469369e-03 + j-4.63847103e-03, 5.07957566e-03 + j-5.31457168e-03, 4.05992171e-03 + j-5.63971464e-03, 2.92371403e-03 + j-5.60022960e-03, 1.73734321e-03 + j-5.22589071e-03, 5.70618496e-04 + j-4.58648188e-03, -5.13205305e-04 + j-3.78295716e-03, -1.45126731e-03 + j-2.93650629e-03, -2.19355915e-03 + j-2.17285059e-03, -2.71003394e-03 + j-1.60607209e-03, -2.99218860e-03 + j-1.32301939e-03

n = 17, 2.11830448e-03 + j-5.93561170e-03, 1.02258539e-03 + j-6.23581414e-03, -2.42696925e-04 + j-6.13775326e-03, -1.48179384e-03 + j-5.64995999e-03, -2.63166195e-03 + j-4.81468380e-03, -3.63537009e-03 + j-3.70282298e-03, -4.44636293e-03 + j-2.40602480e-03, -5.02918214e-03 + j-1.02924723e-03, -5.36015412e-03 + j3.21128397e-04, -5.42898460e-03 + j1.55456075e-03, -5.23838124e-03 + j2.59320748e-03

n = 18, -2.33178243e-03 + j-6.47067949e-03, -3.51271006e-03 + j-6.18920599e-03, -4.62697386e-03 + j-5.51358821e-03, -5.60125503e-03 + j-4.50117914e-03, -6.36816454e-03 + j-3.23716746e-03, -6.87164917e-03 + j-1.82603766e-03, -7.07115435e-03 + j-3.83450218e-04, -6.94309009e-03 + j1.00979564e-03, -6.48399744e-03 + j2.27392847e-03, -5.71130973e-03 + j3.34066786e-03, -4.66253249e-03 + j4.15601522e-03

n = 19, -6.08211873e-03 + j-6.14106719e-03, -6.61590667e-03 + j-5.30169096e-03, -7.09711655e-03 + j-4.07004474e-03, -7.45522295e-03 + j-2.54967256e-03, -7.63824824e-03 + j-8.85152552e-04, -7.61428962e-03 + j7.69004672e-04, -7.37312957e-03 + j2.26291314e-03, -6.92666993e-03 + j3.46603104e-03, -6.30704396e-03 + j4.27629063e-03, -5.56135064e-03 + j4.62784682e-03, -4.74576241e-03 + j4.49692718e-03

n = 20, -9.62217511e-03 + j-4.87254000e-03, -8.95492970e-03 + j-3.16614869e-03, -7.95620903e-03 + j-9.91089548e-04, -6.68913296e-03 + j1.27248863e-03, -5.22925346e-03 + j3.45202062e-03, -3.65935032e-03 + j5.39177931e-03, -2.06326552e-03 + j6.96279377e-03, -5.20733992e-04 + j8.07357158e-03, 8.64480585e-04 + j8.67257958e-03, 2.02618318e-03 + j8.75191802e-03, 2.92297213e-03 + j8.34436655e-03

n = 21, -1.15604798e-02 + j-2.14466984e-03, -9.56176565e-03 + j2.21342946e-04, -7.31815483e-03 + j2.16923005e-03, -4.84175174e-03 + j3.97190136e-03, -2.23872122e-03 + j5.34778732e-03, 1.68904523e-05 + j6.17775950e-03, 2.12519299e-03 + j6.41757342e-03, 4.08503982e-03 + j6.09793258e-03, 5.81406054e-03 + j5.31688673e-03, 7.24885964e-03 + j4.22017545e-03, 8.34921023e-03 + j2.98682419e-03

∂Iin,k∂Vm= 1Mn=0M-1e-jkω0tn∂iin(tn)∂Vm

- ∂Iin,k∂Vm= 122n=021e-jkω0tn*0.041568*ej0.2759

k = −5

m = −5 to 5:

-5: -1.65506210e-05 + j-1.13536878e-02
-4: 4.51519995e-04 + j-7.66732348e-07
-3: -2.11852701e-05 + j7.11806342e-07
-2: -4.75525794e-06 + j-6.12679676e-06
-1: -1.34425429e-06 + j-2.54014719e-06
0: -1.87527366e-06 + j-1.14670565e-06
1: -1.93863744e-06 + j-1.04685655e-06
2: -1.75637035e-06 + j-8.05201980e-07
3: -1.62840099e-06 + j-5.35805817e-07
4: -1.61837259e-06 + j-3.03921755e-07
5: -1.55656286e-06 + j-1.36841629e-07

k = −4

-5: -2.83815841e-04 + j-1.32880132e-05
-4: -1.65506210e-05 + j-9.08302458e-03
-3: 3.70164024e-04 + j-7.66732348e-07
-2: -2.11852701e-05 + j-3.66705421e-06
-1: -4.49634256e-06 + j-6.12679676e-06
0: -1.34425429e-06 + j-2.51638323e-06
1: -1.89128888e-06 + j-1.14670565e-06
2: -1.93863744e-06 + j-1.04164105e-06
3: -1.76367740e-06 + j-8.05201980e-07
4: -1.62840099e-06 + j-5.37509707e-07
5: -1.61408376e-06 + j-3.03921755e-07

k = −3

-5: -1.95483560e-05 + j3.84740549e-05
-4: -2.02459870e-04 + j-1.32880132e-05
-3: -1.65506210e-05 + j-6.81236138e-03
-2: 2.88808052e-04 + j-7.66732348e-07
-1: -2.11852701e-05 + j-8.04591477e-06
0: -4.23742717e-06 + j-6.12679676e-06
1: -1.34425429e-06 + j-2.49261927e-06
2: -1.90730411e-06 + j-1.14670565e-06
3: -1.93863744e-06 + j-1.03642555e-06
4: -1.77098445e-06 + j-8.05201980e-07
5: -1.62840099e-06 + j-5.39213597e-07

k = −2

-5: -1.09512166e-06 + j6.50265483e-06
-4: -1.95483560e-05 + j3.40951943e-05
-3: -1.21103898e-04 + j-1.32880132e-05
-2: -1.65506210e-05 + j-4.54169817e-03
-1: 2.07452081e-04 + j-7.66732348e-07
0: -2.11852701e-05 + j-1.24247753e-05
1: -3.97851179e-06 + j-6.12679676e-06
2: -1.34425429e-06 + j-2.46885531e-06
3: -1.92331933e-06 + j-1.14670565e-06
4: -1.93863744e-06 + j-1.03121005e-06
5: -1.77829150e-06 + j-8.05201980e-07

k = −1

-5: -1.08424552e-06 + j1.97689620e-06
-4: -1.35403704e-06 + j6.50265483e-06
-3: -1.95483560e-05 + j2.97163338e-05
-2: -3.97479272e-05 + j-1.32880132e-05
-1: -1.65506210e-05 + j-2.27103497e-03
0: 1.26096110e-04 + j-7.66732348e-07
1: -2.11852701e-05 + j-1.68036359e-05
2: -3.71959641e-06 + j-6.12679676e-06
3: -1.34425429e-06 + j-2.44509135e-06
4: -1.93933456e-06 + j-1.14670565e-06
5: -1.93863744e-06 + j-1.02599455e-06

k = 0

-5: -2.03863349e-06 + j1.36298707e-06
-4: -1.08424552e-06 + j2.00066016e-06
-3: -1.61295242e-06 + j6.50265483e-06
-2: -1.95483560e-05 + j2.53374732e-05
-1: 4.16080439e-05 + j-1.32880132e-05
0: -1.65506210e-05 + j-3.71770153e-07
1: 4.47401389e-05 + j-7.66732348e-07
2: -2.11852701e-05 + j-2.11824964e-05
3: -3.46068103e-06 + j-6.12679676e-06
4: -1.34425429e-06 + j-2.42132739e-06
5: -1.95534978e-06 + j-1.14670565e-06

k = 1

-5: -1.80402971e-06 + j1.31364140e-06
-4: -2.02261826e-06 + j1.36298707e-06
-3: -1.08424552e-06 + j2.02442412e-06
-2: -1.87186780e-06 + j6.50265483e-06
-1: -1.95483560e-05 + j2.09586126e-05
0: 1.22964015e-04 + j-1.32880132e-05
1: -1.65506210e-05 + j2.27029143e-03
2: -3.66158323e-05 + j-7.66732348e-07
3: -2.11852701e-05 + j-2.55613570e-05
4: -3.20176565e-06 + j-6.12679676e-06
5: -1.34425429e-06 + j-2.39756343e-06

k = 2

-5: -1.65493230e-06 + j9.91810635e-07
-4: -1.80402971e-06 + j1.31885690e-06
-3: -2.00660304e-06 + j1.36298707e-06
-2: -1.08424552e-06 + j2.04818808e-06
-1: -2.13078319e-06 + j6.50265483e-06
0: -1.95483560e-05 + j1.65797521e-05
1: 2.04319986e-04 + j-1.32880132e-05
2: -1.65506210e-05 + j4.54095463e-03
3: -1.17971803e-04 + j-7.66732348e-07
4: -2.11852701e-05 + j-2.99402175e-05
5: -2.94285027e-06 + j-6.12679676e-06

k = 3

-5: -1.55867941e-06 + j7.14246319e-07
-4: -1.64762525e-06 + j9.91810635e-07
-3: -1.80402971e-06 + j1.32407240e-06
-2: -1.99058781e-06 + j1.36298707e-06
-1: -1.08424552e-06 + j2.07195204e-06
0: -2.38969857e-06 + j6.50265483e-06
1: -1.95483560e-05 + j1.22008915e-05
2: 2.85675958e-04 + j-1.32880132e-05
3: -1.65506210e-05 + j6.81161784e-03
4: -1.99327775e-04 + j-7.66732348e-07
5: -2.11852701e-05 + j-3.43190781e-05

k = 4

-5: -1.57079086e-06 + j4.82517773e-07
-4: -1.55867941e-06 + j7.12542429e-07
-3: -1.64031820e-06 + j9.91810635e-07
-2: -1.80402971e-06 + j1.32928789e-06
-1: -1.97457259e-06 + j1.36298707e-06
0: -1.08424552e-06 + j2.09571600e-06
1: -2.64861395e-06 + j6.50265483e-06
2: -1.95483560e-05 + j7.82203098e-06
3: 3.67031929e-04 + j-1.32880132e-05
4: -1.65506210e-05 + j9.08228104e-03
5: -2.80683746e-04 + j-7.66732348e-07

k = 5

-5: -1.53432315e-06 + j3.11536272e-07
-4: -1.57507969e-06 + j4.82517773e-07
-3: -1.55867941e-06 + j7.10838539e-07
-2: -1.63301114e-06 + j9.91810635e-07
-1: -1.80402971e-06 + j1.33450339e-06
0: -1.95855736e-06 + j1.36298707e-06
1: -1.08424552e-06 + j2.11947996e-06
2: -2.90752933e-06 + j6.50265483e-06
3: -1.95483560e-05 + j3.44317043e-06
4: 4.48387900e-04 + j-1.32880132e-05
5: -1.65506210e-05 + j1.13529442e-02

Rk= Ik- Y0Vk, k= -N.. N

δk,m= 1 if k=m0 if k ≠m

Jk,m= ∂Itot,k∂Vm=∂Fk∂Vm= ∂Iin,k∂Vm+ Y0δk,m

In our case k = m

Jk,m= ∂Itot,k∂Vm=∂Fk∂Vm= ∂Iin,k∂Vk+ Y0

Added Y0 = 0.02s then took the inverse Jacobian and get:

k = −5 row

- m = −5: 3.78293015e+01 + j2.14822551e+01

- m = −4: -5.26680890e-01 + j-7.22809700e-01

- m = −3: 3.47152268e-02 + j4.67241382e-02

- m = −2: -1.86330351e-03 + j1.47606871e-02

- m = −1: -6.98641146e-04 + j5.75276060e-03

- m = 0: 2.34516165e-03 + j4.04900104e-03

- m = 1: 2.90018168e-03 + j3.61773754e-03

- m = 2: 3.00340831e-03 + j2.63078333e-03

- m = 3: 3.01850745e-03 + j1.66915568e-03

- m = 4: 3.07344189e-03 + j8.75559123e-04

- m = 5: 2.93889348e-03 + j2.24630151e-04

k = −4 row

- m = −5: 3.07596263e-01 + j4.70384154e-01

- m = −4: 4.14692099e+01 + j1.88323436e+01

- m = −3: -5.82501650e-01 + j-5.45028012e-01

- m = −2: 3.87688053e-02 + j4.44830654e-02

- m = −1: 9.19844419e-04 + j1.58403782e-02

- m = 0: 4.55961689e-04 + j6.20551070e-03

- m = 1: 3.24946107e-03 + j3.73884218e-03

- m = 2: 3.67765251e-03 + j3.11734180e-03

- m = 3: 3.55994070e-03 + j2.09966453e-03

- m = 4: 3.33868327e-03 + j1.13880784e-03

- m = 5: 3.27192008e-03 + j3.06009214e-04

k = −3 row

- m = −5: 8.68338942e-02 + j-1.71230154e-02

- m = −4: 2.95515641e-01 + j3.19873429e-01

- m = −3: 4.48263847e+01 + j1.52728799e+01

- m = −2: -5.69024132e-01 + j-3.47794075e-01

- m = −1: 4.23986030e-02 + j4.27913715e-02

- m = 0: 4.14833101e-03 + j1.60141629e-02

- m = 1: 1.79347705e-03 + j6.23612979e-03

- m = 2: 4.05560185e-03 + j3.08509693e-03

- m = 3: 4.28128506e-03 + j2.33865937e-03

- m = 4: 3.93029995e-03 + j1.37897855e-03

- m = 5: 3.54493167e-03 + j4.55693454e-04

k = −2 row

- m = −5: 1.25645661e-02 + j-8.35908900e-03

- m = −4: 8.10716896e-02 + j-3.15229935e-02

- m = −3: 2.19927314e-01 + j1.73239311e-01

- m = −2: 4.75806317e+01 + j1.08106179e+01

- m = −1: -4.75601781e-01 + j-1.64211294e-01

- m = 0: 4.66862459e-02 + j4.20876998e-02

- m = 1: 7.39030472e-03 + j1.52147267e-02

- m = 2: 3.13177346e-03 + j5.83973654e-03

- m = 3: 4.64133338e-03 + j2.20647130e-03

- m = 4: 4.62846040e-03 + j1.41754334e-03

- m = 5: 4.15350706e-03 + j5.38275356e-04

k = −1 row

- m = −5: 4.67259546e-03 + j-2.26068067e-03

- m = −4: 1.10253352e-02 + j-1.10622769e-02

- m = −3: 7.15006604e-02 + j-4.28665099e-02

- m = −2: 7.88930604e-02 + j6.29095337e-02

- m = −1: 4.94034929e+01 + j5.61367862e+00

- m = 0: -3.12205784e-01 + j-3.36201690e-02

- m = 1: 5.29669145e-02 + j4.14508594e-02

- m = 2: 1.02939647e-02 + j1.35060154e-02

- m = 3: 4.30120801e-03 + j5.03690733e-03

- m = 4: 4.93729636e-03 + j1.20148811e-03

- m = 5: 4.77562965e-03 + j3.97180329e-04

k = 0 row

- m = −5: 5.37929313e-03 + j-4.32572970e-04

- m = −4: 4.18133131e-03 + j-3.33383378e-03

- m = −3: 8.82604581e-03 + j-1.32823352e-02

- m = −2: 5.98925559e-02 + j-4.96418834e-02

- m = −1: -1.07336444e-01 + j2.16984011e-02

- m = 0: 5.00429622e+01 + j5.97757310e-04

- m = 1: -1.11168257e-01 + j1.41344483e-02

- m = 2: 6.14977012e-02 + j3.89181483e-02

- m = 3: 1.25766647e-02 + j1.10775589e-02

- m = 4: 5.14852189e-03 + j3.93017932e-03

- m = 5: 5.01515031e-03 + j1.16953319e-04

k = +1 row

- m = −5: 4.75251673e-03 + j-1.07311332e-03

- m = −4: 5.24336844e-03 + j-1.58944456e-03

- m = −3: 3.48652897e-03 + j-4.20867593e-03

- m = −2: 6.05876427e-03 + j-1.47990491e-02

- m = −1: 4.88837869e-02 + j-5.19733488e-02

- m = 0: -3.00473547e-01 + j6.75834941e-02

- m = 1: 4.94038144e+01 + j-5.61254960e+00

- m = 2: 8.30176481e-02 + j-2.79140846e-02

- m = 3: 7.08754030e-02 + j3.27044662e-02

- m = 4: 1.40495483e-02 + j8.19600820e-03

- m = 5: 5.68607316e-03 + j2.59824895e-03

k = +2 row

- m = −5: 4.02212320e-03 + j-9.92061343e-04

- m = −4: 4.46166744e-03 + j-2.08064442e-03

- m = −3: 4.87267169e-03 + j-2.67009308e-03

- m = −2: 2.54446764e-03 + j-4.84328830e-03

- m = −1: 3.01210554e-03 + j-1.54744853e-02

- m = 0: 4.02421109e-02 + j-5.15515193e-02

- m = 1: -4.57184943e-01 + j1.93902528e-01

- m = 2: 4.75812098e+01 + j-1.08096665e+01

- m = 3: 2.30945421e-01 + j-1.41444016e-01

- m = 4: 7.86935313e-02 + j2.24468711e-02

- m = 5: 1.47773990e-02 + j5.11943536e-03

k = +3 row

- m = −5: 3.47294173e-03 + j-8.37392958e-04

- m = −4: 3.70739048e-03 + j-1.79260850e-03

- m = −3: 3.98083903e-03 + j-2.96035403e-03

- m = −2: 4.18764439e-03 + j-3.59889793e-03

- m = −1: 1.47158034e-03 + j-5.16304744e-03

- m = 0: -1.54398291e-05 + j-1.52843142e-02

- m = 1: 3.40899702e-02 + j-5.04622202e-02

- m = 2: -5.45935152e-01 + j3.71647264e-01

- m = 3: 4.48271201e+01 + j-1.52721646e+01

- m = 4: 3.11647030e-01 + j-2.92831417e-01

- m = 5: 8.32351921e-02 + j9.29158933e-03

k = +4 row

- m = −5: 3.21957616e-03 + j-6.64980298e-04

- m = −4: 3.19331079e-03 + j-1.49146556e-03

- m = −3: 3.26424450e-03 + j-2.46034736e-03

- m = −2: 3.26218086e-03 + j-3.65859804e-03

- m = −1: 3.26998396e-03 + j-4.26891113e-03

- m = 0: 3.94165033e-04 + j-5.14946161e-03

- m = 1: -2.74554038e-03 + j-1.43066665e-02

- m = 2: 2.93434192e-02 + j-4.99363459e-02

- m = 3: -5.56976866e-01 + j5.62500510e-01

- m = 4: 4.14700235e+01 + j-1.88318523e+01

- m = 5: 3.26663405e-01 + j-4.48861634e-01

k = +5 row

- m = −5: 2.90094321e-03 + j-5.51802849e-04

- m = −4: 2.95778932e-03 + j-1.20832672e-03

- m = −3: 2.81731883e-03 + j-1.98577400e-03

- m = −2: 2.65633667e-03 + j-2.92184329e-03

- m = −1: 2.40380967e-03 + j-4.05348965e-03

- m = 0: 2.26186180e-03 + j-4.56157724e-03

- m = 1: -5.40995178e-04 + j-4.78517714e-03

- m = 2: -4.88642048e-03 + j-1.26915694e-02

- m = 3: 2.49694665e-02 + j-4.99416624e-02

- m = 4: -5.00610840e-01 + j7.34165795e-01

- m = 5: 3.78303016e+01 + j-2.14817725e+01

Now Solve for voltage update:

∆V= -J-1R

| K | ΔVj,k(complex) |
| --- | --- |
| -5, 9.79790180e-07 + j9.45479905e-07 -4, 1.70418470e-06 + j-2.68837726e-08 -3, -6.54292483e-06 + j-8.79709824e-06 -2, -4.23444997e-05 + j2.00306533e-04 -1, 8.70702897e-05 + j-5.57987280e-04 0, 1.26169357e-05 + j-1.12736247e-06 1, 1.03964069e-05 + j-7.30289617e-05 2, -6.13796312e-05 + j-1.86256255e-04 3, -4.78188295e-06 + j8.39663477e-06 4, 1.09391169e-06 + j-7.28038833e-07 5, 5.51304467e-07 + j-9.12195810e-07 |  |

Vj,mnew= Vj,mold+∆Vj,m

m, V_j,m^new

-5, 9.79790180e-07 + j9.45479905e-07

-4, 1.70418470e-06 + j-2.68837726e-08

-3, -6.54292483e-06 + j-8.79709824e-06

-2, -4.23444997e-05 + j2.00306533e-04

-1, 2.90507029e-03 + j4.93120127e-02

0, 1.26169357e-05 + j-1.12736247e-06

1, -2.80760359e-03 + j-4.99430290e-02

2, -6.13796312e-05 + j-1.86256255e-04

3, -4.78188295e-06 + j8.39663477e-06

4, 1.09391169e-06 + j-7.28038833e-07

5, 5.51304467e-07 + j-9.12195810e-07

2nd iteration:

ω0​tn= 2πnM

vj,newtn= k = -NNVj,new,k​ejkω0​tn​

n = 0,  -0.00000064 + j-0.00061922

n = 1,   0.02817469 + j-0.00221472

n = 2,   0.05404682 + j-0.00363027

n = 3,   0.07548081 + j-0.00474999

n = 4,   0.09071364 + j-0.00548242

n = 5,   0.09850441 + j-0.00576634

n = 6,   0.09824820 + j-0.00557705

n = 7,   0.09002545 + j-0.00493141

n = 8,   0.07457052 + j-0.00388684

n = 9,   0.05317823 + j-0.00253308

n = 10,  0.02758279 + j-0.00098160

n = 11, -0.00017598 + j0.00064355

n = 12, -0.02790239 + j0.00221379

n = 13, -0.05340711 + j0.00360513

n = 14, -0.07466656 + j0.00470784

n = 15, -0.08997045 + j0.00543441

n = 16, -0.09805837 + j0.00572579

n = 17, -0.09823627 + j0.00555663

n = 18, -0.09044807 + j0.00493853

n = 19, -0.07528599 + j0.00391937

n = 20, -0.05394716 + j0.00257904

n = 21, -0.02814901 + j0.00102405

plug back into i_in(Vj,n), recompute DFT to get I_in,k values

k, I_in,k (complex)

-5, 1.64660397e-08 + j-2.30880597e-09

-4, 2.10801754e-08 + j-4.18958938e-08

-3, -3.03811621e-07 + j3.53032339e-08

-2, 1.00566829e-06 + j4.20731033e-06

-1, 1.11801402e-04 + j-6.37309237e-06

0, 2.38359442e-07 + j5.14751558e-10

1, 1.13147469e-04 + j-6.57841019e-06

2, 4.76028959e-07 + j-4.27922898e-06

3, -2.70133845e-07 + j7.74804994e-09

4, 3.23304092e-08 + j1.86635574e-08

5, 8.81526805e-09 + j-3.93458839e-09

Find fundamental impedance:

vin​(t)=Rs​i​in(tn)+vj​(tn)

n=0,  5.65370574e-03 + j-9.44448701e-04

n=1,  3.37143170e-02 + j-2.52342529e-03

n=2,  5.89950230e-02 + j-3.89587777e-03

n=3,  7.93894266e-02 + j-4.94916008e-03

n=4,  9.32119218e-02 + j-5.59662512e-03

n=5,  9.93477427e-02 + j-5.78320754e-03

n=6,  9.73569191e-02 + j-5.49288901e-03

n=7,  8.74990968e-02 + j-4.75370796e-03

n=8,  7.06691425e-02 + j-3.63388820e-03

n=9,  4.82779140e-02 + j-2.23024084e-03

n=10, 2.21225331e-02 + j-6.56522242e0-04

n=11, -5.74165057e-03 + j9.64052875e-04

n=12, -3.31400105e-02 + j2.50520299e-03

n=13, -5.79306733e-02 + j3.84636173e-03

n=14, -7.81555048e-02 + j4.88229718e-03

n=15, -9.21820286e-02 + j5.53059522e-03

n=16, -9.88374350e-02 + j5.73738846e-03

n=17, -9.75230919e-02 + j5.48273134e-03

n=18, -8.82823703e-02 + j4.78394603e-03

n=19, -7.18066651e-02 + j3.69444488e-03

n=20, -4.93895894e-02 + j2.29915695e-03

n=21, -2.28400525e-02 + j7.09296230e-04

k, V_in,k

-5,  0.00000000e+00 + j7.04e-11

-4,  0.00000000e+00 + j0.00e+00

-3,  0.00000000e+00 + j5.37e-11

-2,  0.00000000e+00 + j0.00e+00

-1,  0.00000000e+00 + j5.00000000e-02

0, -3.15e-19      + j0.00e+00

1,  0.00000000e+00 - j5.00000000e-02

2,  0.00000000e+00 + j0.00e+00

3,  0.00000000e+00 - j5.37e-11

4,  0.00000000e+00 + j0.00e+00

5,  0.00000000e+00 - j7.04e-11

Zin=Vin,1Iin,1= -j5.00*10-2V 1.13147469*10-04 + j-6.57841019*10-06

Zin≈25.6-j440.4 Ω

ZOUT=Vj,1Iin,1= -2.80760359e-03 + j-4.99430290e-02V 1.13147469*10-04 + j-6.57841019*10-06

ZOUT≈0.8464-j441.35 Ω